AFSM Diagram and Description

1. AFSM State Diagram

2. State Descriptions

| State | Description |
| --- | --- |
| Start | All hardware is initialized (servos, ultrasonic sensor, color sensor, LDR, LED), the scan servo is centred, and all timers are reset. Unconditionally transitions to Explore once initialisation is complete. |
| Explore | Normal exploration mode. The robot walks forward (Forword sub-state, full speed) while continuously scanning for obstacles (Scan sub-state, always on). The TCS3200 color sensor checks for the red target all the time. A 2-minute explore timer starts on entry. Exit: red_detected → Target Detected; explore_timer >= 2 min → Low Charge. |
| Low Charge | Simulated low-battery mode entered after 2 minutes of exploration without finding the red target. The robot continues exploring at reduced speed (Forward, lower speed) with Scan still active(decrease frequency slightly). LDR monitoring for high-intensity light is now enabled. Red target detection remains active. Exit: LDR > threshold → Charging; red_detected → Target Detected. |
| Charging | Simulated charging state. The robot stops all movement and remains stationary. Monitors LDR continuously. Obstacle avoidance is NOT active in this state, and the LED steady on. Exit: charging_timer >= 1 min & light still present → Explore; light interrupted before 1 min → Low Charge. |
| Target Detected | The end state with the highest priority. Triggered when the TCS3200 color sensor detects a red patch. The robot stops all movement immediately, and the on-board LED blinks continuous. No outgoing transitions. |

| State | Description |
| --- | --- |
| Scan | Uses the HC-SR04 ultrasonic sensor on a scanning servo to measure distances in three directions: FD (front, 90°), LD (left, 30°), and RD (right, 150°). Always active during Explore and Low Charge. Feeds distance data to the obstacle avoidance decision logic. FD >= 15 cm & LD/RD clear → Forward; obstacle detected → Turn or Veer; all directions blocked → Recovery. |
| Forward | Default walking gait using four leg servos. Lowest priority behaviour. Full speed in Explore, reduced speed in Low Charge. Activated as a parallel sub-state by Explore and Low Charge. Exit: obstacle → Turn/Veer ; stuck → Recovery. |
| Turn Left | 90-degree left turn manoeuvre. Triggered when an obstacle is detected ahead and the left side has more clearance (LD > RD). After completing the turn, the scan re-checks the forward path. Exit: path clear → Forward/Scan; still blocked → Veer Left/Right; turn_count >= 3 → Recovery. |
| Turn Right | 90-degree right turn manoeuvre. Triggered when an obstacle is detected ahead and the right side has more clearance (RD > LD). After completing the turn, the scan re-checks the forward path. Exit: path clear → Forward/Scan; still blocked → Veer Left/Right; turn_count >= 3 → Recovery. |
| Veer Left | 15–30 degree slight left turn. Triggered when the right side has less clearance after a Turn Left, or when a minor adjustment is needed (RD > LD with obstacle ahead). This is a smoother, smaller correction than a full turn. Exit: veering complete → Scan/Forward; still blocked → Turn; turn_count >= 3 → Recovery. |
| Veer Right | 15–30 degree slight right turn. Triggered when the left side has less clearance after a Turn Right, or when a minor adjustment is needed (LD > RD with obstacle ahead). This is a smoother, smaller correction than a full turn. Exit: veering complete → Scan/Forward; still blocked → Turn; turn_count >= 3 → Recovery. |
| Recovery | Safety state triggered when the robot is stuck: three consecutive failed avoidance attempts (turn_count >= 3), or all three ultrasonic directions are blocked (LD < 10 & RD < 10 & FD < 10), or any direction is dangerously close (LD < 5 or RD < 5 or FD < 5). The robot reverses for approximately 1 second, then performs a random turn. Exit: recovery complete & path clear → Scan/Forward; recovery_count >= 3 → Fault Stop (awaiting human intervention). |

3. Transition Table

3.1 Mission State Transitions

| Source | Target | Condition | Description |
| --- | --- | --- | --- |
| Start | Explore | init_complete | Hardware initialised, servos centred, timers reset. |
| Explore | Low Charge | explore_timer >= 2 min | 2-minute exploration timer expired without red target. |
| Explore | Target Detected | red_detected | TCS3200 detects red patch. Immediate stop + LED blink. |
| Low Charge | Charging | LDR > threshold | High-intensity light detected — robot stops to simulate charging. |
| Low Charge | Target Detected | red_detected | TCS3200 detects red patch in low-battery mode. |
| Charging | Explore | charging_timer >= 1 min & light_present | Full 1-minute charge under continuous light. |
| Charging | Low Charge | light_interrupted (timer < 1 min) | Light removed before charge complete. |

3.2 Obstacle Avoidance Transitions

| Source | Target | Condition | Description |
| --- | --- | --- | --- |
| Explore | Forward | default entry (parallel) | Explore activates Forward at full speed. |
| Explore | Scan | always on (parallel) | Explore activates Scan for continuous ultrasonic detection. |
| Low Charge | Forward | default entry, lower speed (parallel) | Low Charge activates Forward at reduced speed. |
| Low Charge | Scan | always on, lower frequency (parallel) | Low Charge activates Scan. |
| Scan | Turn Left | FD < 15 cm & LD > RD | Obstacle ahead, left side has more clearance. |
| Scan | Turn Right | FD < 15 cm & RD > LD | Obstacle ahead, right side has more clearance. |
| Scan | Veer Left | obstacle & RD > LD (minor adjustment) | Slight left correction needed. |
| Scan | Veer Right | obstacle & LD > RD (minor adjustment) | Slight right correction needed. |
| Scan | Recovery | LD<10 & RD<10 & FD<10, or LD<5 or RD<5 or FD<5, or turn_count>=3 | All directions blocked, dangerously close, or repeated failures. |
| Turn Left | Scan | complete turning | Turn finished, re-scan environment. |
| Turn Left | Recovery | turn_count >= 3 | Three consecutive failed left turns. |
| Turn Left | Veer Right | RD > LD | After turn, right side needs minor correction. |
| Turn Right | Scan | complete turning | Turn finished, re-scan environment. |
| Turn Right | Recovery | turn_count >= 3 | Three consecutive failed right turns. |
| Turn Right | Veer Left | LD > RD | After turn, left side needs minor correction. |
| Veer Left | Scan | complete veering | Veering finished, re-scan environment. |
| Veer Left | Recovery | turn_count >= 3 | Repeated veer failures. |
| Veer Right | Scan | complete veering | Veering finished, re-scan environment. |
| Veer Right | Recovery | turn_count >= 3 | Repeated veer failures. |
| Recovery | Scan | recovery complete & path clear | Backup + random turn completed, path now clear. |
| Recovery | Fault Stop | recovery_count >= 3 | Robot is completely stuck — awaiting human intervention. |
| *Any obstacle state | Forward | path clear | When path is confirmed clear, return to default forward movement. |

Note: If LD = RD, the robot turns or veers randomly. All obstacle avoidance states share the turn_count counter, which resets to 0 when the robot returns to Forward. Recovery has its own recovery_count.

4. Priority & Timers

4.1 Priority Table

| Priority | Behaviour | States | Note |
| --- | --- | --- | --- |
| 1 (Highest) | Target Detection | Target Detected | Red detected → immediate stop, preempts everything. |
| 2 | Obstacle Avoidance | Scan, Turn Left, Turn Right, Veer Left, Veer Right, Recovery | Active in Explore and Low Charge only (NOT in Charging). Internal priority: Recovery > Turn > Veer. |
| 3 | Charging Logic | Charging, Low Charge | LDR-based light detection and simulated charging. |
| 4 (Lowest) | Normal Movement | Forward | Default walking gait, most easily interrupted. |

4.2 Timers

| Timer | Duration | Trigger | Action |
| --- | --- | --- | --- |
| Explore Timer | 2 minutes | Entry to Explore | Timer expires → transition to Low Charge. |
| Charging Timer | 1 minute | Entry to Charging | Timer expires & light present → return to Explore; light interrupted before 1 min → return to Low Charge. |
| LED Blink Timer | 500 ms toggle | Entry to Target Detected | Non-blocking LED toggle using ticks_ms(). |
| Turn Count | count >= 3 | Each failed avoidance attempt | Triggers Recovery state. Resets on return to Forward. |
| Recovery Count | count >= 3 | Each Recovery entry | Triggers Fault Stop (awaiting human intervention). |

5. Pseudocode

The pseudocode below describes the four main mission states, following the style shown in the assignment brief.

5.1 Explore State

if State == "Explore"
    while explore_timer != 2 min AND target != found
        # Parallel: Forward (full speed) + Scan (always on)
        if object_detection == False
            go forward (full speed)
        else
            avoid_obstacle()  # Turn / Veer / Recovery
    if target == found
        change state to "Target Detected"
    else
        reset explore timer to 0
        change state to "Low Charge"

5.2 Low Charge State

if State == "Low Charge"
    while target != found
        # Parallel: Forward (reduced speed) + Scan (always on) + LDR check
        if LDR > threshold
            change state to "Charging"
        if object_detection == False
            go forward (reduced speed)
        else
            avoid_obstacle()  # Turn / Veer / Recovery
    if target == found
        change state to "Target Detected"

5.3 Charging State

if State == "Charging"
    # Robot is stationary, no obstacle avoidance
    while charging_timer != 1 min
        if light interrupted
            change state to "Low Charge"
    # charging_timer == 1 min AND light still present
    reset explore timer to 0
    change state to "Explore"

5.4 Target Detected State

if State == "Target Detected"
    stop all movement
    while True
        blink LED every 500 ms  # non-blocking
    # Terminal state — no further transitions